3GPP TSG SA WG3 Security — S3#22	S3-020031

25 - 28 February 2002

Bristol, UK

3GPP TSG-SA WG2 meeting #22	Tdoc S2-020326

Phoenix, USA, 14 - 18 January 2002

Title:	Liaison Statement on "Prefix allocation for IPv6 stateless address autoconfiguration"

Source:	SA2

To:	T1, T2, SA3, SA5, CN1, CN3

Cc:	SA4, CN, T

Contact Person:

#### Name:	Juan-Antonio Ibanez

E-mail Address:	Juan-Antonio.X.Ibanez@erv.ericsson.se

Attachments:	S2-020244, S2-020245, S2-020246 and S2-013330

1. Description:

SA2 would like to inform the above-mentioned groups that during its plenary meeting #22, SA2 has agreed that a unique IPv6 prefix shall be allocated to every primary PDP context when using the IPv6 stateless address autoconfiguration procedure. No particular requirements on IPv6 stateful address allocation have been discussed. The attached CRs to 23.060 have been approved, which introduce the changes to the IPv6 stateless address autoconfiguration procedure in GPRS from release 99 onwards. Also attached, the contribution S2-013330, which was presented in SA2#21, explains the reasons for this change. IPv6 prefix allocation will in particular allow UEs to freely change their interface identifier, for instance for privacy reasons (RFC 3041), without the need for the network to be involved. This also implies that the actual interface identifier used by the UE will not be known to the PLMN.

SA2 kindly asks the above-mentioned groups to consider these changes in their work and investigate possible impacts on their respective specifications. SA2 however has not identified any direct impacts on SA3 and SA5 specifications. Note also that these changes do not affect the SGSN.

2. Actions:

T1, T2, SA3, SA5, CN1 and CN3:
To consider these changes in their work and investigate possible impacts on their respective specifications.

3. Date of Next SA2 Meetings:

SA2 #23	18th – 22nd February 2002		Sophia-Antipolis, France

SA2 #24	22nd – 26th April 2002			Madrid, Spain