3GPP TSG SA WG3-LI	S3LI02_056

29 – 31 January 2002

Amsterdam, Netherlands

# SA3LI01-1??

# To:		SA3LI, Amsterdam, 29-31 January 2002

# Title:		PDP Context Modification, Continue Record

### Date:		6 December 2001

From:		Koen Jaspers

Introduction

Document 23.060 describes in clause 9.2 besides the activation and deactivation also the PDP Context Modification. The delivery of  PDP Context Modification to the LEA’s is a requirement. The current versions of 33.107v500 and 33.108v020/030 do not describe this functionality.

During the work shop of SA3LI in Ratingen there was a general agreement to add this in a future version of the documents. This document proposes the changes for 33.108v030.

(CR’s for 33.107 will be generated in parallel with this document)

# Current clause

The current clause 6.5.1.3 is a place holder for Continue Records

#### 6.5.1.3	CONTINUE record information

For  futher  study.

# Proposal

The following change is proposed to clause 6.5.1.3.

The evident changes are differently marked from the specific ones.

#### 6.5.1.3	Continue record information

The CONTINUE record is used to convey events of during packet-data communication PDP Context active.

The CONTINUE record shall be triggered when:

- An active PDP context is modified;

Table 6-8: PDP Context Modification CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| observed PDP address | C | The observed address after modification Provide to identify the: - - static address requested by the intercept subject’s MS, and allocated by the Network for a successful PDP context activation. - - address allocated dynamically by the network to the intercept subject MS in association with a PDP context activation (i.e., address is sent by the Network in an Activate PDP Context Accept) for a successful PDP context activation procedure when the PDP Context activation request does not contain a static PDP address. - - address offered by the network in association with a network-initiated PDP context activation request when the intercept subject’s MS accepts the network-initiated PDP context activation request. |
| event type | C | Provide the PDP Context Modification event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| access point name | C | - Provide to identify the: - - packet data network to which the intercept subject requested to be connected when the intercept subject’s MS is successful at performing a PDP context activation procedure (MS to Network). - - access point of the packet data network that requested to be connected to the MS when the intercept subject’s MS accepts a network-initiated PDP context activation (Network to MS). |
| PDP type | C | Provide to describe the PDP type of the observed PDP address. The PDP Type defines the end user protocol to be used between the external packet data network and the MS. |
| initiator | C | Provide to indicate whether the PDP context activation is network-initiated, intercept-subject-initiated, or not available. |
| communication identifier | M | Provide network identifier. |
| correlation number | C | Provide to uniquely identify the PDP context delivered to the LEMF used to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when authorized, to identify location information for the intercept subject’s MS. |
| QOS | C | Provide to identify the QOS parameters. |