# 5 Circuit-switch domain

For the US J-STD-25 is mandaotory!

## 5.1	Identifiers

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the data, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers are defined in the subsections below.

For the delivery of CC and IRI the MSC provide correlation numbers and target identities to the HI2 and HI3. The correlation number is unique per call and is used to correlate CC with IRI and the different IRI’s of one call.

### 5.1.1	Lawful interception identifier

For each target identity related to an interception measure, the authorized NWO/AP/SvP operator shall assign a special Lawful Interception Identifier (LIID), which has been agreed between the LEA and the NWO/AP/SvP.

Using an indirect identification, pointing to a target identity makes it easier to keep the knowledge about a specific interception target limited within the authorized NWO/AP/SvP operators and the handling agents at the LEA.

The LIID is a component of the CC delivery procedure and of the IRI records. It shall be used within any information exchanged at the handover interfaces HI2 and HI3 for identification and correlation purposes.

The LIID format shall consist of alphanumeric characters. It might for example, among other information, contain a lawful authorization reference number, and the date, when the lawful authorization was issued.

The authorized NWO/AP/SvP shall either enter a unique LIID for each target identity of the interception subject or a single LIID for multiple target identities all pertaining to the same interception subject.

If more than one LEA intercepts the same target identity, there shall be unique LIIDs assigned relating to each LEA.

### 5.1.2	Network identifier

The network identifier (NID) is a mandatory parameter; it should be internationally unique. It consists of the following two identifiers.

1)	NWO/AP/SvP- identifier (mandatory):
Unique identification of network operator, access provider or service provider.

2)	Network element identifier NEID (optional):
The purpose of the network element identifier is to uniquely identify the relevant network element carrying out the LI operations, such as LI activation, IRI record sending, etc.

### 5.1.3	Correlation number

The Correlation Number is unique per call and used for the following purposes:

- correlate CC with IRI,

- correlate different IRI records within one call.

## 5.2	Performance, reliability, and quality

### 5.2.1	Timing

As a general principle, within a telecommunication system, intercept related information (IRI), if buffered, should be buffered for as short a time as possible.

NOTE:	If the transmission of intercept related information fails, it may be buffered or lost.

Subject to national requirements, the following timing requirements shall be supported:

- Each IRI data record shall be sent by the delivery function to the LEMF over the HI2 within seconds of the detection of the triggering event by the IAP at least 95% of the time.

- Each IRI data record shall contain a time-stamp, based on the intercepting nodes clock, that is generated following the detection of the IRI triggering event.

## 5.3	Security aspects

Security is defined by national requirements.

## 5.4	Quantitative aspects

- The number of target interceptions supported is a national requirement.

The area of Quantitative Aspects addresses the ability to perform multiple, simultaneous interceptions within a provider’s network and at each of the relevant intercept access points within the network.  Specifics related to this topic include:

- The ability to access and monitor all simultaneous communications originated, received, or redirected by the interception subject.

- The ability for multiple LEAs (up to five) to  monitor, simultaneously, the same interception subject while maintaining unobtrusiveness, including between agencies.

- The ability of the network to simultaneously support a number of separate (i.e., multiple interception subjects) legally authorized interceptions within its service area(s), including different levels of authorization for each interception, including between agencies (i.e., IRI only, or IRI and communication content).

## 5.5	IRI for CS

Intercept related information will in principle be available in the following phases of a data transmission:

- At call attempt when the target identity becomes active

- At the end of a call, when the target identity becomes inactive.

- At certain times when relevant information are available (SMS;…

In addition, information on non-transmission related actions of a target constitute IRI and is sent via HI2, e.g. information on subscriber controlled input.

The intercept related information (IRI) may be subdivided into the following categories:

1.	Control information for HI2 (e.g. correlation information).

2.	Basic call information, for a standard call between two parties.

- The events defined in ref [19] are used to generate records for the delivery via HI2.

There are eight different event types received at DF2/MF. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2/MF and record type sent to the LEMF.

| - Event | - IRI Record Type |
| --- | --- |
| - Call establishment | - BEGIN |
| - Answer | - CONTINUE |
| - Supplementary service | - CONTINUE |
| - Handover | - CONTINUE |
| - Release | - END |
| - Location update | - REPORT |
| - Subscriber controlled input | - REPORT |
| - SMS | - REPORT |

Table 5-1: Structure of the records for GSM (CS)

A set of information is used to generate the records. The records used transmit the information from mediation function to LEMF. This set of information can be extended in MSC/VLR or GMSC or DF2/MF, if this is necessary in a specific country. The following table gives the mapping between information received in event and information sent in records.

| parameter | definition | ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber) | PartyInformation/msISDN |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber) | PartyInformation/imsi |
| observed IMEI | Target Identifier with the IMEI of the target subscriber (monitored subscriber), it must be checked for each call over the radio interface | PartyInformation/imei |
| event type | Description which type of event is delivered: Establishment, Answer, Supplementary service, Handover, Release, SMS, Location update, Subscriber controlled input | UMTS-CS-Event |
| event date | Date of the event generation in the MSC/VLR or GMSC | timestamp |
| event time | Time of the event generation in the MSC/VLR or GMSC |  |
| dialed number | Dialed number before digit modification, IN-modification etc. | PartyInformation (=originating) /DSS1-parameters / calledpartynumber |
| connected number | number of the answering party | PartyInformation/supplementary-Services-Info |
| other party address | Directory number of the other party for originating calls Calling party for terminating calls | PartyInformation (=terminating) / calledpartynumber PartyInformation / callingpartynumber |
| call direction | Information if the monitored subscriber is calling or called e.g. MOC/MTC or originating/ terminating in or/out | intercepted-Call-Direct |
| CID | Unique number for each call sent to the DF, to help the LEA, to have a correlation between each call and the IRI (combination of Interception Node ID and the correlation number). | communicationIdentifier |
| lawful interception identifier | Unique number for each surveillance warrant | LawfulInterceptionIdentifier |
| cell id | Cell number of the target; for the location information | locationOfTheTarget |
| location area code | Location-area-code of the target defines the Location Area in a PLMN |  |
| basic service | Information about Tele service or bearer service. | PartyInformation/DSS1-parameters-codeset-0 |
| supplementary service | Supplementary services used by the target e.g. CF, CW, ECT | PartyInformation/Supplementary-Services |
| forwarded to number | Forwarded to number at CF | PartyInformation/calledPartyNumber (party-Qualifier indicating forwarded-to-party) |
| call release reason | Call release reason of the target call | release-Reason-Of-intercepted-Call |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |
| SCI | Non call related Subscriber Controlled Input (SCI) which the MSC/VLR receives from the ME | PartyInformation /sciData |

Table 5-2 Description of parameters

NOTE : LIID parameter must be present in each record sent to the LEMF.

### 5.5.1	Events and information

This clause describes the information sent from the Delivery Function (DF)/MF Mediation Function to the Law Enforcement Monitoring Facility (LEMF) to support Lawfully Authorized Electronic Surveillance (LAES).   The information is described as records and information carried by a record. This focus is on describing the information being transferred to the LEMF.

The IRI events and data are encoded into records as defined in the Table 1 Mapping between  Events and HI2 records type and Annex B.3 Intercept related information (HI2) [1]. IRI is described in terms of a ‘causing event’ and information associated with that event.  Within each IRI Record there is a set of events and associated information elements to support the particular service.

The communication events described in Table 5-1: Mapping between UMTS Events and HI2 record type and Table 5-2: Mapping between Events information and IRI information convey the basic information for reporting the disposition of a communication.  This clause describes those events and supporting information.

Each record described in this clause consists of a set of parameters. Each parameter is either:

mandatory (M)	- required for the record,

conditional (C)	- required in situations where a condition is met (the condition is given in the Description

optional (O)	- provided at the discretion of the implementation.

The information to be carried by each parameter is identified.  Both optional and conditional parameters are considered to be OPTIONAL syntactically in ASN.1 Stage 3 descriptions.  The Stage 2 inclusion takes precedence over Stage 3 syntax.

#### 5.5.1.1	REPORT record information

The REPORT record is used to report non-communication related subscriber actions (events).

The REPORT record shall be triggered when:

- A location update of the interception subscriber is detected

- A  subscriber controlled input (SCI) is detected

- the intercept subject’s mobile station sends an SMS-Mobile Originated (MO) communication. Dependent on national requirements, the triggering event shall occur either when the 3G MSC receives the SMS from the target MS or, when the 3G MSC receives notification that the SMS-Centre successfully received the SMS.

- the intercept subject’s mobile station receives a SMS Mobile-Terminated (MT) communication.  Dependent on national requirements, the triggering event shall occur either when the 3G MSC receives the SMS from the SMS-Centre or, when the 3G MSC receives notification that the target MS successfully received the SMS.

Table 5-3: Location update REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | O | Provide location update event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | M | New location information for the intercept subject’s MS. |

Table 5-4: Subscriber Controlled Input REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| Observed IMSI |  |  |
| event type | O | Provide SCI event type. |
| Event date | M | Provide the date and time the event is detected. |
| Event time |  |  |
| network identifier | M | Shall be provided. |
| Lawful intercept identifier | M | Shall be provided. |
| Location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| SCI | M | Subscriber Controlled Input |

Table 5-5: SMS-MO and SMS-MT Communication REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| event type | O | Provide SMS event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| SMS | M | Provide to deliver SMS content, including header which is sent with the SMS-service. |

#### 5.5.1.2	BEGIN record information

The BEGIN record is used to convey the first event of the intercepted call

The BEGIN record shall be triggered when:

- a call attempt of/for the intercepted subscriber is detected at the 3G MSC

Table 5-6: Call establishment BEGIN Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | O | Provide Call establishment event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| Dialed number | C | Dialed number (before modification) |
| Other party address | C | Directory number of the other party for originating calls Calling party for terminating calls |
| Call direction | M | Terminating or originating |
| Redirection number | C | Number invoked the call forwarding towards the target |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the call delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| Basic Service | C | Tele service or bearer service |
| Supplementary service | C | Supplementary service used by the target |

#### 5.5.1.3	CONTINUE record information

The CONTINUE record is used to convey events during a call

The CONTINUE record shall be triggered when:

- the called party answers

- a supplementary service occurs

- a handover occurs

Table 5-7: Answer CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | O | Provide Answer event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| Dialed number | C | Dialed number (before modification) |
| Other party address | C | Directory number of the other party for originating calls Calling party for terminating calls |
| Call direction | O | Terminating or originating |
| Redirection number | O | Number invoked the call forwarding towards the target |
| Connected number | C | Number of the answering party |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the call delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| Basic Service | O | Tele service or bearer service |
| Supplementary service | O | Supplementary service used by the target |

Table 5-8: Supplementary Service CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | O | Provide Supplementary Service event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| Dialed number | O | Dialed number (before modification) |
| Other party address | O | Directory number of the other party for originating calls Calling party for terminating calls |
| Call direction | O | Terminating or originating |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the call delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| Basic Service | O | Tele service or bearer service |
| Supplementary service | M | Supplementary service used by the target |
| Forwarded to number | C | Forwarded to number at CF |

Table 5-8: Handover CONTINUE Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | O | Provide Handover event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the call delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |

#### 5.5.1.4	END record information

The END record is used to convey the last event of a intercepted call.

The END record shall be triggered when:

- the call is released

Table 5-10: Call release END Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | O | Provide Call release event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| Dialed number | O | Dialed number (before modification) |
| Other party address | O | Directory number of the other party for originating calls Calling party for terminating calls |
| Call direction | O | Terminating or originating |
| network identifier | M | Shall be provided. |
| correlation number | M | Provide to uniquely identify the call delivered to the LEMF and to correlate IRI records with CC. |
| lawful intercept identifier | M | Shall be provided. |
| location information | C | Provide, when available, to identify location information for the intercept subject’s MS. |
| Basic Service | O | Tele service or bearer service |
| Call release reason | O | Call release reason of the target call |

## Content of Communication for CS

The port HI3 shall transport the content of the communication (CC) of the intercepted telecommunication service to the LEMF. The content of communication shall be presented as a transparent en-clair copy of the information flow during an established, frequently bi-directional, communication of the interception subject. It may contain voice or data. A target call has two directions of transmission associated with it, to the target, and from the target. Two communication channels to the LEMF are needed for transmission of the content of communication (stereo transmission).

The network does not record or store the content of communication.

### 5.6.1	Delivery of content of communication

The transmission media used to support the HI3 port shall be standard ISDN calls, based on 64 kbit/s circuit switched bearer connections. The CC links are set up on demand to the LEMF. The LEMF constitutes an ISDN DSS1 user function, with an ISDN DSS1 basic or primary rate access. It may be locally connected to the target switching node, or it may be located somewhere in the target network or in another network, with or without a transit network in between (in this case other standards as ISDN could be used). For network signalling, the standard ISDN user part shall be used. No modifications of the existing ISDN protocols shall be required. Any information needed for LI, like to enable correlation with the IRI records of a call, can be inserted in the existing messages and parameters, without the need to extend the ETSI standard protocols for the LI application.

For each LI activation, a fixed LEMF address is assigned; this address is, within the present document, not used for any identification purposes; identification and correlation of the CC links is performed by separate, LI specific information.

The functions defined in the ISDN user part standard, Version 1 (ETSI ISUP V1) are required as a minimum within the target network and, if applicable, the destination and transit networks, especially for the support of:

- Correlation of HI3 information to the other HI port's information, using the supplementary service user-to-user signalling 1 implicit (UUS1) or subaddressing.

The bearer capability used for the CC links is 64 kbit/s unrestricted digital information; this type guarantees that the information is passed transparently to the LEMF. No specific HLC parameter value is required.

The CC communication channel is a one-way connection, from the NWO/AP/SvP's/DF3/MF  to the LEMF, the opposite direction is not switched through in the delivering node of the target.

The scenario for delivery of the content of communication is as follows:

- At call attempt initiation, for one 64 kbit/s bi-directional target call, two ISDN delivery calls are established from the MF to the LEMF. One call offers the content of communication towards the target identity (CC Rx call/channel), the other call offers the content of communication from the target identity (CC Tx call/channel).

- During the establishment of each of these calls, appropriate checks are made.

- The MF passes during call set up, within the signalling protocol elements of the CC link the LIID and the CID to the LEMF. The LEMF uses this information to identify the target identity and to correlate between the IRI and CC.

- At the end of a call attempt, each delivery call associated with that call attempt shall be released by the MF.

### 5.6.2	Control information for Circuit switched content of communication

The delivery calls shall use unmodified standard ISDN protocols (DSS1, ISDN user part). The table below summarizes specific settings of parameters for the CC links. The User-to-User service 1 parameter or subaddressing is used during call set up  to transmit LI-specific control information. This information is carried transparently and delivered to the specific LEMF remote user.

To identify the delivered information, including correlating the delivery calls with the IRI records, parameters 1 to 3 and 5 shall be included in the call set up. Parameters 6 to 9 specify settings of further relevant information. Other parameters of the ISDN protocols shall correspond to normal basic calls.

Table 5-11: Definition of HI3 specific signalling information; UUS1 coding details.

| No. | Used information element of CC link signalling protocol | Information | Purpose |
| --- | --- | --- | --- |
| 1 | CLI-Parameter with attribute "network provided" |  | LEMF can check identity of origin of call. |
| 2 | UUS1 - parameter | lawful interception identifier (LIID); | Identifier, identifying target identity |
| 3 | UUS1 - parameter | Communication Identifier (CID), | Identifier, identifying specific call of target identity |
| 4 | UUS1 - parameter | Not used | Not used |
| 5 | UUS1 - parameter | Direction indication (communication from / towards target / combined (mono)) | Signal from (Tx) / towards (Rx) target identity or combined |
| 6 | UUS1 - parameter | bearer capability of target call | (optional) |
| 7 | closed user group interlock code | closed user group interlock code | Supplementary Service CUG Security measure at set up of the CC link |
| 8 | basic service (BS) | basic service (BS) of CC link: 64 kbit/s unrestricted | Guarantee transparent transmission of CC copy from MF to LEMF |
| 9 | ISDN user part forward call indicators parameter | ISDN user part preference indicator: "ISDN user part required all the way" | Guarantee transparent transmission of UUS1 and other supplementary services information |
| 10 | ISDN user part optional forward call indicators parameter | Connected line identity request parameter: requested | sending of the connected number by the destination network |

Parameters 2 and 3 are also present in the IRI records, for correlation with the CC links. Parameter 5 indicates in case of separate transmission of each communication direction, which part is carried by a CC link. Parameter 6, the basic service of the target call, can be used by the LEMF for processing of the CC signal, e.g. to apply compression methods for speech signals, in order to save storage space. Parameter 7 contains the CUG of the LEA. It is optionally used at set up the CC link to the LEA. Parameter 8, the basic service of the CC link, is set to "64 kbit/s unrestricted": All information of the Rx, Tx channels can be transmitted fully transparently to the LEA. The setting of the ISDN user part indicator guarantees, that the services supporting the LI CC link delivery are available for the complete CC link connection.

The MF uses en-bloc dialling, i.e. there exists only one message in forward direction to the LEA.

NOTE:	The LEMF should at reception of the set up message not use the alerting state, it should connect immediately, to minimize time delay until switching through the CC links. Not all networks will support such a transition. Exceptionally, it may be necessary to send an alerting message before the connected message.

The maximum length of the user information parameter can be more than the minimum length of 35 octets i.e. the network transmitting the CC links shall support the standard maximum size of 131 octets for the UUS1 parameter.

The User-to-User service 1 parameter cannot be discarded by the ETSI ISUP procedures: The only reason, which would allow the ISUP procedures to discard it would be, if the maximum length of the message carrying UUS1 would be exceeded. With the specified amount of services used for the CC links, this cannot happen.

The signalling messages of the two CC channels (stereo mode) carry the same parameter values, except for the direction indication.

See annex E for the ASN.1 definition of the UUS1 LI specific content of the UUS1 parameter and sub-addressing.

### 5.6.3	Exception Handling

#### 5.6.3.1	Failure of CC links

If a CC link cannot be set up, a certain number of repeat attempts during a certain period of time shall be made.

NOTE:	Typical values are 3 tries during 10 seconds.

In case of a delay or total failure to transmit the content of communication, the target call is handled fully independent of the CC link, the CC information gets lost.

### 5.6.4	Security requirements at the interface port HI3

The process of access verification and additional (optional) authentication between the MF and the LEMF shall not delay the set up of the CC.

For the protection and access verification of the content of communication delivery call the ISDN supplementary services CLIP, COLP and CUG shall be used when available in the network involved.

Generally any authentication shall be processed before the setup of the CC links between the MF and the LEMF is completed. If this is technically not feasible the authentication may be processed after completion of the CC connection in parallel to the existing connection.

#### 5.6.4.1	LI access verification

The supplementary service CLIP shall be used to check for the correct origin of the delivery call.

NOTE:	When using CLIP, the supplementary service CLIR must not be used.

The supplementary service COLP shall be used to ensure that only the intended terminal on the LEA's side accepts incoming calls from the handover interface (HI).

To ensure access verification the following two checks shall be performed:

- check of calling-line identification presentation (CLIP) at the LEMF; and

- check of connected-line identification presentation (COLP) at the handover interface (HI) (due to the fact that the connected number will not always be transported by the networks involved, there shall be the possibility for deactivating the COLP check for a given interception measure. In addition, the COLP check shall accept two different numbers as correct numbers, i.e. the user provided number and the network provided number. Usually, the user provided number contains a DDI extension).

#### 5.6.4.2	Access protection

- In order to prevent faulty connections to the LEA, the CC links may be set up as CUG calls.

- In this case, the following settings of the CUG parameters should be used:

- Incoming Access:						not allowed;

- Outgoing Access:						not allowed;

- Incoming calls barred within a CUG:	no;

- Outgoing calls barred within a CUG:	yes.

#### 5.6.4.3	Authentication

- In addition to the minimum access verification mechanisms described above, optional authentication mechanisms according to the standard series ISO 9798 "Information technology - Entity authentication - parts 1 to 5" may be used.

- These mechanisms shall only be used in addition to the access verification and protection mechanisms.

# Annex D (normative)

# Interception related information HI2 (CS)

For the US J-STD-25 is mandaotory!

Umts-Circuit-SwitchedHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) threeGPP (4) hi2Cs(3) vesion1 (1)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5)

version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

PartyInformation,

CommunicationIdentifier,

CC-Link-Identifier,

National-Parameters,

IP-value

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version3(3)} -- TS 101 671 Edition 3

Location,

SMS-report

FROM UmtsHI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulintercept(2) threeGPP (4) hi2(1) version-1(1)};

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4)etsi(0)securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2CSDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2CS(3) version-1(1)}

umtsCS-sending-of-IRI  OPERATION ::=

{

ARGUMENT	Umts-Circuit-Switched-IRIContent

ERRORS		{ OperationErrors }

CODE		global:{

threeGPPSUBDomainId hi2(1) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

Umts-Circuit-Switched-IRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

IRI-Parameters		::= SEQUENCE

{

hi2CSDomainId				[0]	OBJECT IDENTIFIER,

iRIversion					[23]	ENUMERATED

{

version1(1),

...

},

lawfulInterceptionIdentifier		[1] LawfulInterceptionIdentifier OPTIONAL,

-- This identifier is associated to the target.

-- called CallIdentifier in Edition 1 of the document

communicationIdentifier				[2] CommunicationIdentifier OPTIONAL,

--used to uniquely identify an intercepted call.

-- called CallIdentifier in Edition 1 of the document

timeStamp							[3] TimeStamp,

-- date and time of the event triggering the report.)

locationOfTheTarget					[8] Location OPTIONAL,

-- location of the target subscriber

partyInformation 					[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party (Originating, Terminating or

-- forwarded party), the identiy(ies) of the party and all the information

-- provided by the party.

cC-Link-Identifier					[10] CC-Link-Identifier OPTIONAL,

release-Reason-Of-Intercepted-Call	[11] OCTET STRING (SIZE (2)) OPTIONAL,

-- Release cause coded in [31] format.

-- This parameter indicates the reason why the

-- intercepted call cannot be established or why the intercepted call has been

-- released after the active phase.

national-Parameters					[16] National-Parameters OPTIONAL,

umts-Cs-Event						[33] Umts-Cs-Event OPTIONAL

}

Umts-Cs-Event::=ENUMERATED

{

call-establishment				(1),

answer					(		(2),

supplementary-Service			(3),

handover						(4),

release							(5),

sMS								(6),

location-update					(7),

subscriber-Controled-Input 		(8),

...

}

END

# Annex E (normative)

# Definition of the UUS1 content associated and sub addressing to the CC link

For the US J-STD-25 is mandaotory!

For the transport of the correlation information and the identifiers accompanying the CC-links there are two options:

E.1.	Use of the UUS1 ...

E.2.	Use of the SUB ...

## E.1 ASN.1 description of the UUS1 content associated to the CC link

HI3CCLinkData

{ ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) threeGPP(4) hi3CS (4)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

LawfulInterceptionIdentifier,

CommunicationIdentifier,

CC-Link-Identifier

FROM

HI2Operations

{ ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) hi2 (1) version2 (2)};

Direction-Indication,

Service-Information,

Bearer-capability

FROM

HI3CCLinkData

{ ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) hi3 (2) cclinkLI (4) version2 (2)}

UUS1-Content	::= SEQUENCE

{

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

communicationIdentifier			[2] CommunicationIdentifier,

direction-Indication	[4] Direction-Indication,

bearer-capability		[5] Bearer-capability

service-Information		[7] Service-Information OPTIONAL,

...

}

END -- HI3CCLinkData

## E.2 Use of sub-address and calling party number to carry correlation information

### E.2.1	Introduction

The calling party number, the calling party sub-address (CgP Sub) and the called party sub-address (CdP Sub) are used to carry correlation information.

### E.2.2	Sub-address options

The coding of a sub-address information element is given in EN 300 356. The following options shall be chosen:

Table E.1

| Option | Value |
| --- | --- |
| Type of sub-address | user specified |
| Odd/even indicator | employed for called party sub-address when no national parameters are used |

### E.2.3	Sub-address coding

The coding of sub-address information shall be in accordance with EN 300 356.

#### E.2.3.1	BCD Values

The values 0-9 shall be BCD coded according to their natural binary values. The hexadecimal value F shall be used as a field separator. This coding is indicated in table E.2:

Table E.2

| Item | BCD representation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit 4 | Bit 3 | Bit 2 | Bit 1 |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 |
| Field separator | 1 | 1 | 1 | 1 |

When items are packed two to an octet, the least significant item shall be coded by mapping bit 4 to bit 8, bit 3 to bit 7, etc.

#### E.2.3.2	Field order and layout

Fields shall be presented into the sub-address in the following order:

Table E.3:	Fields in the Called Party Sub-address

| Order | #### Field |
| --- | --- |
| 1 | Operator-ID |
| 2 | CIN |
| 3 | Not used |
| 4 | National Parameters |

Table E.4 	Fields in the Calling Party Subaddress

| Order | Field |
| --- | --- |
| 1 | Lawful Interception Identifier (LIID) |
| 2 | Direction |
| 3 | Service Octets |

Each field noted above shall be included, whether empty or not, and a field separator shall separate each field. When a field is empty, that shall be indicated by two consecutive field separators. There shall be a field separator after the final field, too.

The Service Octets as available shall always be mapped into octets 19 to 23 of the Calling Party Sub-address, as appropriate. If one of the parameters TMR, BC or HLC is not available, the octet shall be fill with 'FF' hex. If Mobile Teleservice Code is not available, octet 23 shall not be transmitted. If Mobile Teleservice Code and Mobile Bearer Service Code are not available, octets 22 and 23 shall not be transmitted.

The tables E.5 represent called party sub-address and table E.6 calling party sub-address with the maximum length of the identifiers:

Table E.5:	 Called Party Sub-address

| Bits |  |  |  |  |  |  |  |  | Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |
| Called party sub-address identifier |  |  |  |  |  |  |  |  | 1 |
| Length of called party sub-address contents |  |  |  |  |  |  |  |  | 2 |
| Type of sub-address = user specified, odd/even indicator |  |  |  |  |  |  |  |  | 3 |
| Operator-ID  |  |  |  | Operator-ID  |  |  |  |  | 4 |
| Operator-ID  |  |  |  | Operator-ID  |  |  |  |  | 5 |
| Field separator |  |  |  | Operator-ID  |  |  |  |  | 6 |
| CIN  |  |  |  | CIN  |  |  |  |  | 7 |
| CIN  |  |  |  | CIN  |  |  |  |  | 8 |
| CIN  |  |  |  | CIN  |  |  |  |  | 9 |
| CIN  |  |  |  | CIN  |  |  |  |  | 10 |
|  |  |  |  | Field separator |  |  |  |  | 11 |
|  |  |  |  |  |  |  |  |  | 12 |
|  |  |  |  |  |  |  |  |  | 13 |
|  |  |  |  |  |  |  |  |  | 14 |
|  |  |  |  |  |  |  |  |  | 15 |
|  |  |  |  |  |  |  |  |  | 16 |
|  |  |  |  |  |  |  |  |  | 17 |
|  |  |  |  |  |  |  |  |  | 18 |
|  |  |  |  |  |  |  |  |  | 19 |
|  |  |  |  |  |  |  |  |  | 20 |
|  |  |  |  |  |  |  |  |  | 21 |
|  |  |  |  |  |  |  |  |  | 22 |
|  |  |  |  |  |  |  |  |  | 23 |

Table E.6:	 Calling Party Sub-address

| Bits |  |  |  |  |  |  |  |  |  | Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 |  | 4 | 3 | 2 | 1 |  |  |
| Calling party subaddress identifier |  |  |  |  |  |  |  |  | 1 |  |
| Length of calling party subaddress contents |  |  |  |  |  |  |  |  | 2 |  |
| Type of subaddress = user specified, odd/even indicator according to the amount of BCD-digits |  |  |  |  |  |  |  |  | 3 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 4 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 5 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 6 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 7 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 8 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 9 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 10 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 11 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 12 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 13 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 14 |  |
| LIID  |  |  |  | LIID  |  |  |  |  | 15 |  |
| Field separator |  |  |  | LIID  |  |  |  |  | 16 |  |
| Field separator |  |  |  | Direction |  |  |  |  | 17 |  |
| spare |  |  |  | spare |  |  |  |  | 18 |  |
| Q.763 TMR (note 1) |  |  |  |  |  |  |  |  | 19 |  |
| Q.931 BC octet 3 (note 2) |  |  |  |  |  |  |  |  | 20 |  |
| Q.931 HLC octet 4 (note 3) |  |  |  |  |  |  |  |  | 21 |  |
| Mobile Bearer Service Code (note 4) |  |  |  |  |  |  |  |  | 22 |  |
| Mobile Teleservice Code (note 5) |  |  |  |  |  |  |  |  | 23 |  |

NOTE 1:	if available, the Transmission Medium Requirement according to EN 300 356. If not available, the value is 'FF' hex.

NOTE 2:	if available, only octet 3 of the Bearer Capability I.E. according to EN 300 403  If not available, the value is 'FF' hex.

NOTE 3:	if available, only octet 4 of the High Layer Compatibility I.E. according to EN 300 403. If not available, the value is 'FF' hex.

NOTE 4:	if available, the Mobile Bearer Service Code according to ETS 300 974, clause 14.7.10. If not available, the octets 22 and 23 shall not be transmitted.

NOTE 5:	if available, the Mobile Teleservice Code according to ETS 300 974, clause 14.7.9. If not available, the octet 23 shall not be transmitted).

### E.2.4	Field coding

Each field shall employ decimal coding, except for the Service Octets (octets 19-23 of the CgP Sub) and the octets reserved for national use (octets 16-23 of the CdP Sub). Other values are not permitted.

#### E.2.4.1	Direction

The direction field shall be coded as follows:

Table E.7

| Indication | Value |
| --- | --- |
| Mono mode (combined signal) (historic) | 0 |
| CC from target | 1 |
| CC to target | 2 |

#### E.2.4.2	Coding of the Calling Party Number

The Network Element Identifier (NEID) shall be carried by the calling party number information element. The coding shall be as follows, depending on the type of network access (note 1):

Numbering plan identification:	ISDN/telephony numbering plan (Recommendation E.164)

Nature of address:	International number (in case of  ISUP signalling)

Type of number:	International number (in case of  DSS1 signalling)

Screening indicator	Network provided (in case ISUP signalling)

Screening indicator	User-provided, not screened (in case of DSS1 signalling, note 2)

Presentation indicator:	Presentation allowed

NOTE 1:	Usually, the IIF respectively the Mediation Function is connected to the network by links using Signalling System Number 7 and ISDN User Part (ISUP), whereby the parameters are coded according to [5]. But in some cases, the IIF respectively the Mediation Function may be connected via a Basic Rate Access or a Primary Rate Access using D-Channel signalling, whereby the parameters are coded  according to [6].

NOTE 2:	The network will perform screening, i.e. the number will arrive at the LEMF as 'user-provided, verified and passed'. A network provided number shall also be accepted at the LEMF.

### E.2.5	Length of fields

The length of the identifiers is variable. The minimum and maximum length of each field shall be as given in the table below:

Table E.8

| Field | Minimum length (decimal digits) | Maximum length (decimal digits) | Maximum length (Half-Octets) | I.E. |
| --- | --- | --- | --- | --- |
| Operator ID | 2 | 5 | 5+1 | CdP Sub |
| CIN | 6 | 8 | 8+1 | CdP Sub |
| LIID | 2 | 25 | 25+1 | CgP Sub |
| Direction | 1 | 1 | 1+1 | CgP Sub |
| Service Octets |  |  | 10 | CgP Sub |