3GPP SA3-LI #10 	Tdoc S3LI02_074r1

Orlando/FL (USA); April 9 – 11, 2002

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.108 |  |  |  | CR | Xxx |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 0.7.3 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Addition of SMS type information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Alcatel S.A. (laurent.perrine@alcatel.fr) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | Security |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2002-04-04 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | TS 33.108 already includes in the ASN.1 definitions a parameter to support the capability of reporting the SMS-initiator. However, this parameter is missing in other parts of the specification. This contribution proposes additions to TS 33.108 to achieve consistency within the document. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Add parameter SMS-initiator in table 2 and table 6.5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Inconsistency in 33.108 regarding sMS-initiator parameter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks” feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ for the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

- With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification, which are not relevant to the change request.

*** First Modification ***

Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation (party-identiity) |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation (party-identity) |
| observed IMEI | Target Identifier with the IMEI of the target subscriber (monitored subscriber) | partyInformation (party-identity) |
| observed PDP address | PDP address used by the target.. | partyInformation (services-data-information) |
| event type | Description which type of event is delivered: PDP Context Activation, PDP Context Deactivation,GPRS Attach, etc. | gPRSevent |
| event date | Date of the event generation in the xGSN | timeStamp |
| event time | Time of the event generation in the xGSN |  |
| access point name | The APN of the access point | partyInformation (services-data-information) |
| PDP type | This field describes the PDP type as defined in TS GSM 09.60, TS GSM 04.08, TS GSM 09.02 | partyInformation (services-data-information) |
| initiator | This field indicates whether the PDP context activation, deactivation, or modification is MS directed or network initiated. | initiator |
| correlation number | Unique number for each PDP context delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |
| lawful interception identifier | Unique number for each lawful authorization. | lawfulInterceptionIdentifier |
| location information | - This field provides the service area identity, RAI and/or location area identity that is present at the SGSN at the time of event record production. | - locationOfTheTarget |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |
| failed context activation reason | This field gives information about the reason for a failed context activation of the target subscriber. | gPRSOperationErrorCode |
| failed attach reason | This field gives information about the reason for a failed attach attempt of the target subscriber. | gPRSOperationErrorCode |
| service center address | This field identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. | serviceCenterAddress |
| QOS | This field indicates the Quality of Service associated with the PDP Context procedure. | qOS |
| context deactivation reason | This field gives information about the reason for context deactivation of the target subscriber. | gPRSOperationErrorCode |
| network identifier | Operator ID plus SGSN or GGSN address. | networkIdentifier |
| iP assignment | Observed PDP address is statically or dynamically assigned. | iP-assignment |
| SMS originating address | Identifies the originator of the SMS message. | DataNodeAddress |
| SMS terminating address | Identifies the intended recipient of the SMS message. | DataNodeAddress |
| SMS initiator | Indicates whether the SMS is MO or MT | sms-initiator |

*** Next Modification ***

Table 6-5: SMS-MO and SMS-MT Communication REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| event type | C | Provide SMS event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| SMS originating address | O | Provide, if available, to identify the originating and destination address of the SMS message. |
| SMS destination address |  |  |
| location information | C | Provide, when authorized, to identify location information for the intercept subject’s MS. |
| SMS | C | Provide to deliver SMS content, including header which is sent with the SMS-service. |
| service center address | C | Provide to identify the address of the relevant SMS-C server. If SMS content is provided, this parameter is optional. |
| SMS initiator | M | Indicates whether the SMS is MO, MT, or undefined. |